Test Summary

Latchup

|  | IF input (GHz, dBm) | LO input (GHz, dBm) | RF Output (GHz, dBm) |
| --- | --- | --- | --- |
| Typical Board | 1, -10 | 8, 19 | 7, -19; 9, -20 |

Transient

|  | IF input (GHz, dBm) | LO input (GHz, dBm) | RF Output (GHz, dBm) |
| --- | --- | --- | --- |
| Typical Board | 1, -10 | 8, 19 | 7, -19; 9, -20 |

Brief Test Setup

- Connect all relevant equipment to the chassis (2x rf gen)

- Connect the eval board to the appropriate equipment – see connection diagram (2x rf gens, spectrum analyzer)

- Connect the PXIe4081 to the beam’s TTL Signal

- If latchup, heat die to 150℃. If transient, keep at room temperature

- Use the LabVIEW vi to configure the run information and the rf sources

- Record video of spectrum analyzer

- Irradiate beam

- End beam irradiation

- End LabVIEW power supply reading and the python register read

- Evaluate the data

- Offload data collection folder to separate pc to read

Equipment

- NI PXI 1095 Chassis

  - 8880 controller

  - 4081 dmm

- 2x Signal Generators (Hittite Sig Gens)

- Spectrum Analyzer

- ADMV1530

- >3x rf cables

- >1x 2.4mm to 2.92 FF RF connector

- >2x Right Angle MF RF connector

- >1x HS-USB-GPIB connectors

- 2 – 3 x GPIB connectors

- Camera

- Omega Thermometer

- Thermocouples

Procedure

NOTES

Initial Configuration

- Ensure the device is powered off

- Connect one RF Hittite Signal Generator labelled “GPIB 27” to IF

- Connect a HS-USB-GPIB connector on the between RF Sig Gen “GPIB 27” to the NI Tester

- Connect one RF Hittite Signal Generator labelled “GPIB 12” to LO

- Connect a GPIB to GPIB connect between RF Sig Gen “GPIB 27” and “GPIB 12”

- Connect the PxIe4081 to the beam’s TTL logic via bnc /banana jack adaptor on the patch panel

- If latchup test, put the heat gun on the part and heat to 150 celcius

- Open LabVIEW on NI Tester

- Go to C:\ Git\Rad-Effects-Test-Software\SEE_PXI_TopBench.lvproj

  - If there is a pop up, just press “load with selected”

- If not running, press white arrow

- Configure the “Data Block”

  - Change values in the “Run Data Input” (Green Block)

    - RUN FOLDER

    - DUT

    - ION

    - BOARD NUMBER

    - EFFECTVE LET

    - TEST TYPE (NOTE ONLY DOING LATCHUP THIS CAMPAIGN)

  - Press “Apply Data” in the “Output Echo” Block (right block)

  - Confirm the values are desired

  - Ensure the “Changes Applied?” indicator is bright green

  - Press the Exit button

- Configure the “RF Block”

  - IF = 1Ghz , -10dBm

    - Select the drop down on “VISA resource name”

    - Select GPIB2::27::INSTR

    - Set “frequency” to 1

    - Set “power” to -10

    - Press “Power On”

    - Press Apply Configuration

    - Check the “RF Block Output Echo” for the configuration

  - LO = 1Ghz , -10dBm

    - Select the drop down on “VISA resource name”

    - Select GPIB2::12::INSTR

    - Set “frequency” to 1

    - Set “power” to 19

    - Press “Power On”

    - Press Apply Configuration

    - Check the “RF Block Output Echo” for the configuration

- Configure the Oscilloscope Block

  - Just open and close it. This functionality will be added later

- Begin Data Collection Block

- Confirm that the data is being collected at the levels you are expecting for a unconfigured board

- Confirm that a folder has been created at the run folder specified during STEP 15 A I

- Review the spectrum analyzer output. If there is a tone at 1GHz greater than -7 dBm, you have configured the part properly

- Prepare the beam for irradiation

- On the LabVIEW script, press “record data toggle” when you want to begin

- Record spectrum analyzer via camera

- Once run is complete, press “record data toggle” on the LabVIEW script

- You have completed a run