Test Summary

Latchup

|  | LO input (GHz, dBm) | IF_HYB input (GHz, dBm) | 3p0 input (V) | 3p0 config’d current (mA) | 1p8_A input (V) | 1p8_A config’d current (mA) | 1p8_B input (V) | 1p8_B config’d current (mA) | Output tone (GHz, dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Board 1 | 13, -5 | 8, -20 | 3.36 | 65 | 2.06 | 198 | 2.085 | 227 | 18, ~-9.4 |
| Board 2 | 13, -5 | 8, -20 | 3.5 | 60 | 2.2 | 185 | 2.24 | 220 | 18, ~-10.0 |

Transient

|  | LO input (GHz, dBm) | IF_HYB input (GHz, dBm) | 3p0 input (V) | 3p0 config’d current (mA) | 1p8_A input (V) | 1p8_A config’d current (mA) | 1p8_B input (V) | 1p8_B config’d current (mA) | Output tone (GHz, dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Board 1 | 13, -5 | 8, -20 | 2.7 | 75 | 1.63 | 220 | 1.628 | 270 | 18, ~-9.0 |
| Board 2 | 13, -5 | 8, -20 | 2.88 | 59 | 1.82 | 173 | 1.84 | 212 | 18, ~-11.2 |

Brief Test Setup

- Connect all relevant equipment to the chassis (2x rf gen, 3x power cards)

- Connect the eval board to the appropriate equipment – see connection diagram (2x rf gen, 3x power cards, spectrum analyzer)

- Use the LabVIEW vi to configure the run information, the power cards, the rf sources

- Use ACE + SDP connector to configure the eval board

- Set up the python register collection script

- Run the LabVIEW power supply reading and the python register read

- Irradiate beam

- End beam irradiation

- End LabVIEW power supply reading and the python register read

- Evaluate the data

- Offload data collection folder to separate pc to read

Equipment

- Keithley Power Supply

- 2x Signal Generators (Hittite Sig Gens)

- Spectrum Analyzer

- ADMV1355 eval board

- 4-5 RF Cables

- 1x 2.4mm to 2.92 FF RF connector

- 5-6x Right Angle MF RF connector

- 2x HS-USB-GPIB connectors

- 2 – 3 x GPIB connectors

Procedure

NOTES

Slot 16 cable has some voltage drop on it (significant enough to account for it)

Initial Configuration

- Ensure the device is powered off

- Connect one RF Hittite Signal Generator to LO

  - GPIB 27

- Connect one RF Hittite Signal Generator to IF_HYB

  - GPIB 12

- Connect a HS-USB-GPIB connector on each Hittite Signal Generator to the NI Tester

  - May be able to get away with a GPIB connector between the two RF Generators

- Connect RF_OUT to Spec Analyzer

- Connect 3V input to the PXIe4139 on Slot 4 of the PXI 1095 tester

- Connect 1P8_B input to the PXIe4139 on Slot 5 of the PXI 1095 tester NOT TYPO

- Connect 1P8_A input to the PXIe4139 on Slot 6 of the PXI 1095 tester NOT TYPO

- Open LabVIEW on NI Tester

- Go to C:\ Git\Rad-Effects-Test-Software\SEE_PXI_TopBench.lvproj

  - If there is a pop up, just press “load with selected”

- If not running, press white arrow

- Configure the “Data Block”

  - Change values in the “Run Data Input (Green Block)

    - RUN FOLDER

    - DUT

    - ION

    - BOARD NUMBER

    - EFFECTVE LET

    - TEST TYPE (NOTE ONLY DOING LATCHUP THIS CAMPAIGN)

  - Press “Apply Data” in the “Output Echo” Block (right block)

  - Confirm the values are desired

  - Ensure the “Changes Applied?” indicator is bright green

  - Press the Exit button

- Configure the “Power Block”

IMPORTANT NOTE: YOU MUST DO STEP C AFTER STEP A AND B, NEVER BEFORE

  - Select “Slot_14_PXIe4139”

    - Select Active Channel 0 (bright green when selected)

    - “voltage level channel 0” should be 3

    - “Channel Name 0” should be “3p0_voltage_input” (DO NOT HIT ENTER, just click out of the box)

      - AVOID USING SPACES AND PERIODS IN NAMES

    - “Aperture Time” should be .001 seconds (could go to .0001 seconds if you want higher resolution)

    - Current Limit should be 3 (Maximum 3)

    - Press Apply Changes

    - Check DCPowerInformantionClusterArray for one object

  - Select “Slot_15_PXIe4139”

    - Select Active Channel 0 (bright green when

    - “voltage level channel 0” should be 1.8

    - “Channel Name 0” should be “1p8_B_voltage_input” (DO NOT HIT ENTER)

      - AVOID USING SPACES AND PERIODS IN NAMES

    - “Aperture Time” should be .001 seconds (could go to .0001 seconds if you want higher resolution)

    - Current Limit should be 3 (Maximum 3)

    - Press Apply Changes

    - Check DCPowerInformationClusterArray for two objects

  - Select “Slot_16_PXIe4139”

    - Select Active Channel 0

    - “voltage level channel 0” should be 1.8

    - “Channel Name 0” should be “1p8_A_voltage_input” (DO NOT HIT ENTER

    - “Aperture Time” should be .001 seconds (could go to .0001 seconds if you want higher resolution)

    - Current Limit should be 3 (Maximum 3)

    - Press “Apply Changes”

    - Check DCPowerInformationClusterArray for three objects

  - Press “Update Changes”

  - When powered on, the device should have the following values in Table ___ and Table ____

  - You may now exit

Latchup

|  | 3p0 supply voltage (V) | 3p0 supply current (mA) | 1p8_A supply voltage (V) | 1p8_A supply current (mA) | 1p8_B supply voltage (V) | 1p8_B supply current (mA) |
| --- | --- | --- | --- | --- | --- | --- |
| Board 1 | 3.36 | 2 | 2.06 | 43 | 2.085 | 10 |
| Board 2 | 3.5 | 2.6 | 2.2 | 43 | 2.24 | 10.7 |
| Board 3 | - | - | - | - | - | - |

Transient

|  | 3p0 supply voltage (V) | 3p0 supply current (mA) | 1p8_A supply voltage (V) | 1p8_A supply current (mA) | 1p8_B supply voltage (V) | 1p8_B supply current (mA) |
| --- | --- | --- | --- | --- | --- | --- |
| Board 1 | 2.7 | 1.5 | 1.63 | 40 | 1.628 | 8 |
| Board 2 | 2.88 |  | 1.82 |  | 1.84 |  |
| Board 3 | - | - | - | - | - | - |

- Configure the “RF Block”

  - IF_HYB = 8Ghz , -20dBm

    - Select the drop down on “VISA resource name”

    - Select GPIB2::27::INSTR

    - Set “frequency” to 8

    - Set “power” to -20

    - Press “Power On”

    - Press Apply Configuration

    - Check the “RF Block Output Echo” for the configuration

  - LO = -5 dBm, 13GHz

    - Select the drop down on “VISA resource name”

    - Select GPIB2::12::INSTR

    - Set “frequency” to 13

    - Set “power” to -5

    - Press “Power On”

    - Press Apply Configuration

    - Check the “RF Block Output Echo” for the configuration

- Configure the Oscilloscope Block

  - Just open and close it. This functionality will be added later

- Begin Data Collection Block

- Confirm that the data is being collected at the levels you are expecting for a unconfigured board

- Confirm that a folder has been created at the run folder specified during STEP 12 A I

- IF ACE IS NOT ALREADY OPEN OR FIRST TIME YOU ARE SETTING UP THIS SPECIFIC DUT, PLEASE DO THE FOLLOWING – IF NOT, REFER TO STEP 20

  - Connect cable to sdp-s

  - Connect sdp-s to ADMV1355 eval board

  - Open ACE in administrator mode (don’t update anything if it gives you prompts)

  - Open the ADMV1355 plugin

    - The power supplies should be the following

| Table: Latchup when board plugin has been opened in ACE but not configured (not under temp) |  |  |  |
| --- | --- | --- | --- |
|  | 3p0 (mA) | 1p8_A (mA) | 1p8_B (mA) |
| Board 1 | 62 | 260 | 217 |
| Board 2 | 75 | 260 | 220 |
| Board 3 | - | - | - |

| Table: Transient when board plugin has been opened in ACE but not configured (not under temp) |  |  |  |
| --- | --- | --- | --- |
|  | 3p0 (mA) | 1p8_A (mA) | 1p8_B (mA) |
| Board 1 | 160 | 400 | 270 |
| Board 2 | 170 | 390 | 275 |
| Board 3 | - | - | - |

  - Select Tools dropdown (left side)

  - Select Macro Tools (should be checked)

  - Press the folder icon that should now be on the right side of the window

  - Open ADMV1355_config.acemacro

    - LOCATION AT “C:\Git\Rad-Effects-Test-Software\ACE MACROS”

    - Press Play

  - The power supplies should be the following when configured (Table ____)

Latchup

| Table: Latchup when board plugin has been configured in ACE (not under temp) |  |  |  |
| --- | --- | --- | --- |
|  | 3p0 (mA) | 1p8_A (mA) | 1p8_B (mA) |
| Board 1 | 65 | 198 | 227 |
| Board 1 | 60 | 185 | 220 |
| Board 3 | - | - | - |

Transient

|  | 3p0 current | 1.8_A current (mA) | 1.8_B current (mA) |
| --- | --- | --- | --- |
| Board 1 | 75 | 220 | 270 |
| Board 2 | 59 | 173 | 212 |
| Board 3 | - | - | - |

  - Review the output tone on the spectrum analyzer and the power supply currents on the NI PXI Tester

Latchup

|  | Frequency (GHz) | Power (dBm) |
| --- | --- | --- |
| Board 1 | 18 | -9.6 to -9.2 |
| Board 2 | 18 | -10.1 to -9.9 |

Transient

|  | Frequency (GHz) | Power (dBm) |
| --- | --- | --- |
| Board 1 | 18 | ~ -9.0 |
| Board 2 | 18 | ~-11.3 to -11.1 |

  - Open Command prompt in in administrator mode

    - Command prompt without admin should be fine though

  - Enter “cd C:\Git\Rad-Effects-Test-Software\Python_Scripts\ADMV” into the prompt

  - Enter “py ADMV1355.py”

  - Press enter for every prompt until “press any key to begin recording”

  - When you reach the “press any key to begin recording” wait until you are ready to collect data with the beam

- IF ACE IS OPEN AND SAME DUT AS PREVIOUS RUN (you have completed this procedure once before up through step 28)

  - Power up the board

  - On the ACE plugin, go to the SDP button

  - Toggle 1,2,3,4 in that order

  - Follow steps 14e – 14m

- Prepare the beam for irradiation

- On the LabVIEW script, press “record data toggle” when you want to begin

- On the Python script, press enter when you want to begin recording

- When the beam is on, write down the time stamp in a text file (you’ll have a time stamp at the beginning of the LabVIEW recording and in the python capture, but this is for being closer to the actual beam time

- Once run is complete, press CTRL + C on the python program

- Once run is complete, press “record data toggle” on the LabVIEW script

- You have completed a run

- POWER DOWN PROCEDURE WHILE KEEPING ACE the ACE Plugin up

  - On the ACE plugin, go to the SDP-S button (left side)

  - Toggle 3,2,1,0 in that order

  - On the LabVIEW VI, go to the power block

  - Select “Slot_16_PXIe4139”

  - Set Voltage to Zero

  - Press “Apply Changes”

  - Press “Update Changes”

  - Select “Slot_15_PXIe4139”

  - Set Voltage to Zero

  - Select “Slot_14_PXIe4139”

  - Set Voltage to Zero

  - Press “Update Changes”

  - Device should now be powered off