Test Summary

Latchup

| Table 1: Overall Configuration in Latchup |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RF input (GHz, dBm) | VDD (V) | IDD (mA) | VSS (V) | ISS (mA) | EN (V) | LS (V) | V1 (V) | V2 (V) | Output tone (GHz, dBm) |
| Typical Board | 1, -5 | 3.6 | >.155 | -3.6 | >.530 | 0 | 0 | 0 | 0 | 1, ??? |

Transient

| Table 2: Overall Configuration in Transient |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RF input (GHz, dBm) | VDD (V) | IDD (mA) | VSS (V) | ISS (mA) | EN (V) | CTRL (V) | V1 (V) | V2 (V) | Output tone (GHz, dBm) |
| Typical Board | 1, -5 | .3 | <.155 | .3 | <.530 | 0 | 0 | 0 | 0 | 1, ??? |

Brief Test Setup

- Connect all relevant equipment to the chassis (1x rf gen, 1x power card)

- Connect the eval board to the appropriate equipment – see connection diagram (1x rf gen, 1x power cards, spectrum analyzer)

- Connect the PXIe4081 to the beam’s TTL Signal

- If latchup, heat die to 135℃. If transient, keep at room temperature

- Use the LabVIEW vi to configure the run information, the power cards, the rf sources

- Use ACE + SDP connector to configure the eval board

- Set up the python register collection script

- Run the LabVIEW power supply reading and the python register read

- Irradiate beam

- End beam irradiation

- End LabVIEW power supply reading and the python register read

- Evaluate the data

- Offload data collection folder to separate pc to read

Equipment

- NI PXI 1095 Chassis

  - 8880 controller

  - 4163 smu card

  - 4081 dmm

- 1x Signal Generators (Hittite Sig Gens)

- Spectrum Analyzer

- ADRF5050

- >2x rf cables

- >1x 2.4mm to 2.92 FF RF connector

- >2x Right Angle MF RF connector

- >1x HS-USB-GPIB connectors

- 2 – 3 x GPIB connectors

- >3x 50 ohm screw terminations

- Camera

- Omega Thermometer

- Thermocouples

Procedure

NOTES

Slot 6 cable has some voltage drop on it (significant enough to account for it)

Initial Configuration

- Ensure the device is powered off

- Connect one RF Hittite Signal Generator to RFC

  - GPIB 27

- Connect a HS-USB-GPIB connector on the Hittite Signal Generator to the NI Tester

- Connect RF2 to Spec Analyzer

- Connect VDD input to Channel 0 on the 4163 (slot 3) of the PXI 1095 tester

- Connect VSS input to Channel 1 on the 4163 (slot 3) of the PXI 1095 tester

- Connect EN input to Channel 2 on the 4163 (slot 3) of the PXI 1095 tester

- Connect LS input to Channel 3 on the 4163 (slot 3) of the PXI 1095 tester

- Connect V1 input to Channel 4 on the 4163 (slot 3) of the PXI 1095 tester

- Connect V2 input to Channel 5 on the 4163 (slot 3) of the PXI 1095 tester

- Connect the PxIe4081 to the beam’s TTL logic via bnc /banana jack adaptor on the patch panel

- Connect GND to the GND on the 4163 (slot 3) of the PXI 1095 tester

  - Can connect the PXIe4081 to the shared ground node, but not required

- If latchup test, put the heat gun on the part and

- Open LabVIEW on NI Tester

- Go to C:\ Git\Rad-Effects-Test-Software\SEE_PXI_TopBench.lvproj

  - If there is a pop up, just press “load with selected”

- If not running, press white arrow

- Configure the “Data Block”

  - Change values in the “Run Data Input” (Green Block)

    - RUN FOLDER

    - DUT

    - ION

    - BOARD NUMBER

    - EFFECTVE LET

    - TEST TYPE (NOTE ONLY DOING LATCHUP THIS CAMPAIGN)

  - Press “Apply Data” in the “Output Echo” Block (right block)

  - Confirm the values are desired

  - Ensure the “Changes Applied?” indicator is bright green

  - Press the Exit button

- Configure the “Power Block”

  - Select “PXIe4163”

    - Select Active Channel 0 through Active Channel 3 (bright green when selected)

    - Refer to Tables 3 and 4 for the voltage necessary for the device and the channel name

      - AVOID USING SPACES AND PERIODS IN NAMES

    - “Aperture Time” should be .001 seconds (could go to .0001 seconds if you want higher resolution)

    - Current Limit should be .05

    - Press Apply Changes

    - Check DCPowerInformantionClusterArray for one object

Latchup

| Table 3: LATCHUP Expected Voltage/Current values along with the channel names of each configured power supply |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VDD (V) | IDD (mA) | VSS (V) | ISS (mA) | EN (V) | LS (V) | V1 (V) | V2 (V) |
| Typical Board | 3.6 | .155 < | -3.6 | .530 < | 0 | 0 | 0 | 0 |
| Channel Name | VDD |  | VSS |  | EN | CTRL | V1 | V2 |

Transient

| Table 4: TRANSIENT Expected Voltage/Current values along with the channel names of each configured power supply |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VDD (V) | IDD (mA) | VSS (V) | ISS (mA) | EN (V) | CTRL (V) | V1 (V) | V2 (V) |
| Typical Board | .3? | < .155 | -.3 | <.530 | 0 | 0 | 0 | 0 |
| Channel Name | VDD |  | VSS |  | EN | CTRL | V1 | V2 |

- Configure the “RF Block”

  - RFC = 1Ghz , -5dBm

    - Select the drop down on “VISA resource name”

    - Select GPIB2::27::INSTR

    - Set “frequency” to 1

    - Set “power” to -5

    - Press “Power On”

    - Press Apply Configuration

    - Check the “RF Block Output Echo” for the configuration

- Configure the Oscilloscope Block

  - Just open and close it. This functionality will be added later

- Begin Data Collection Block

- Confirm that the data is being collected at the levels you are expecting for a unconfigured board

- Confirm that a folder has been created at the run folder specified during STEP 17 A I

- Review the spectrum analyzer output. If there is a tone at 1GHz greater than -7 dBm, you have configured the part properly

- Prepare the beam for irradiation

- On the LabVIEW script, press “record data toggle” when you want to begin

- When the beam is on, write down the time stamp in a text file (you’ll have a time stamp at the beginning of the LabVIEW recording and in the python capture, but this is for being closer to the actual beam time

- Once run is complete, press “record data toggle” on the LabVIEW script

- You have completed a run