- Connect cables to power supplies

  - 3x 4139 cables, 1x 4163 cable

- Connect 1 HMC sig gens

- Connect the RF connections as follows in the diagram

- Connect the banana jacks as follows

- Be sure to connect all ground connections together.

| Black GND | GND connections |
| --- | --- |
| Red 3.3v VDD | 4 VCC (4139 card) |
| Orange 1.5v VDDLDO | 5 VCC (4139 card) |
| Yellow 1.1v VIO | 6 VCC (4139 card) |
| Green 1.1v VDD | CH0 (4163 card) |

- Open the Labview software

  - Configure the Data block as necessary

- Open the power block

Be sure to activate all channels and queue the changes

  - Slot_4

    - Voltage = 3.3v, channel name = Red 5v VDD

  - Slot_5

    - Voltage = 1.5v, channel name = Orange 3.3v VIO

  - Slot_6

    - Voltage = 1.1v, channel name = Yellow 1.8v VDDLDO

  - Slot_3

    - Voltage = 1.1v, channel name = Green 1.1v VDD

- Open the RF Block

  - GPIB 12

    - Name = RF_IN, Freq = 1MHz, Power = 0dBm

- Check unconfigured Supply Currents

- Note!  If the part is pre-configured this could be different, see the configured supply currents if this doesn’t match.

| Red 5v VDD | Orange 3.3v VIO | Yellow 1.8v VDDLDO | Green 1.1v VDD |
| --- | --- | --- | --- |

- Open the script AD4080_SEE at C:\Git\pyadi-iio\examples\Radiation Testing

- Run the script

- Note the configured Supply Currents

| Red 5v VDD | Orange 3.3v VIO | Yellow 1.8v VDDLDO | Green 1.1v VDD |
| --- | --- | --- | --- |
| 16mA | 31mA | 0 | 0 |